Date Prepared:   8/6/10

Material Safety Data Sheet

Section 1: Product and Company Identification

Product Name:          Epoxy 100 Resin (Part B)

Sold By:                     Slide-Lok

11803 N. Saguaro Blvd.

Fountain Hills, AZ 85268

(480) 222-9720

FOR CHEMICAL        Spill, Leak, Fire, Exposure, or Accident, Call 24 hours

EMERGENCY            800-835-1759

Fax Phone:                 480-222-9711

Section 2 A: Product/Ingredient

| Number | Composition | CAS Number | Percent |
| --- | --- | --- | --- |
| P | Compound Ingredient | MIXTURE |  |
| 1 | Polymer of EPICHLORHYDRIN & BISPHENOL A | 025085-99-8 | 70%-80% |
| 2 | ALKYL GLYCIDYL ETHER (C12-C14) | 068609-97-2 | 10%-30% |

Section 2 B: Toxicity Data

| Number | Acute Oral LD50 | Acute Dermal LD50 | Acute Inhalation |
| --- | --- | --- | --- |
| P NO DATA |  | NO DATA | NO DATA |
| 1 | .11.4 G/KG (RAT) | 20 ML/KG (RABBIT) | No death in Saturated air, 8 hours. |

Section 3: Health Information

Eye Contact

Based on product testing, product is moderately irritating to the eyes.

Skin Contact

Moderately irritating to the eyes.

Inhalation

Because of its low volatility, this product is not likely to be an inhalation hazard.

Ingestion

Considered to have a low order of Acute Oral Toxicity.

Signs/Symptoms

Irritation as noted. Skin sensitization (ALLERGY) May be evidenced by rashes, especially hives.

Other Effects

Section 4: Occupational Exposure Limits

| Number | OSHA PEL | ACGIH/TLV | VAPOR PRESSURE |  | OTHER |
| --- | --- | --- | --- | --- | --- |
| P |  |  | 25 C mm Hg | TEMP | WT PERCENT |
| 1 | N/A | N/A | N/A | N/A | N/A |

Section 5: Emergency and First Aid Procedures

Eye Contact

Immediately flush eyes with plenty of water for at least 15 minutes while holding eyelids open. rinse continuously with water while on way to get medical attention.

Skin Contact

Immediately remove contaminated clothing or shoes, wipe excess from skin. Flush skin with water. Follow by washing with soap and water. Do not reuse clothing until thoroughly decontaminated. Leather articles, including shoes, cannot be decontaminated and should be destroyed.

Inhalation

Remove victim to fresh air and provide oxygen if breathing is difficult. Get medical attention if necessary

Ingestion

Do not induce vomiting. In general, no treatment is necessary unless large quantities of product are ingested. However, get medical advice.

Section 6: Physical Data

Boiling Point:  Approximately 500, Degrees F

Melting Point:  N/A

Evaporation Rate:  Not Applicable

Appearance:  Mobile, Viscous Liquid

Specific Gravity: 9.02 lbs. / gal

Solubility (in Water):  Negligible

Vapor Pressure:  .03 at 77, Degrees C

Vapor Density:  Not Applicable

Section 7: Fire and Explosion Hazards

Flash Point and Method: T.C.C., F. 115

Flammable Limits: Lower: N/A    Upper: N/A

Extinguishing Media: Use water for, "ALCOHOL" Foam, Dry chemical or CO2.

Fire Fighting Procedures and Precautions: Material will not burn unless preheated. Do not enter confined fire space without full bunker gear, including a positive pressure NIOSH approved self-contained breathing apparatus. Cool fire exposed containers with water.

Unusual Fire and Explosion Hazards: See Section 11

Section 8: Reactivity

Stability: Stable                                                                      Hazardous Polymerization: Will Not Occur

Conditions and Materials to Avoid

Can react vigorously with strong oxidizing agents, strong Lewis or mineral acids, and strong mineral and organic bases / especially primary and secondary aliphatic amines.

Hazardous Decomposition Products

Carbon Monoxide, Aldehydes, Acids and other organic substances may be formed during combustion or elevated (500 Deg. F) Temperature Degradation.

Section 9: Employee Protection

Respiratory Protection: Not ordinarily required

Avoid contact with eyes. Wear Safety Glasses or goggles as appropriate. Avoid Prolonged or repeated contact with skin. Wear chemical resistant gloves and other clothing as required to minimize contact.

Protective Clothing

Section 10: Environmental Protection

Spill or Leak Procedures: May burn although not readily ignitable. Use Cautious Judgment when cleaning up large spills.

Section 11: Special Precautions

Store in a cool, Dry place with adequate ventilation. Keep away from open flames and high temperatures.

Containers, even those that have been emptied, Can contain hazardous product residues. Wash with soap and water before eating, drinking, smoking, applying cosmetics, or using toilet facilities.

Section 12: Transportation Requirements

Department of Transportation Classification:

CLASS 8 (CORROSIVE MATERIAL), II

D.O.T. PROPER SHIPPING NAME

Polyamines, liquid, corrosive, N.O.S.